Mid-rise vs Mid-tread Quantiser

Jack Li @ AUT

Most uniform quantizers for signed input data can be classified as being of one of two types: mid-riser and mid-tread. The terminology is based on what happens in the region around the value x=0, and uses the analogy of viewing the input-output function of the quantizer as a stairway. Mid-tread quantizers have a zero-valued reconstruction level (corresponding to a tread of a stairway), while mid-riser quantizers have a zero-valued classification threshold (corresponding to a riser of a stairway). Mid-tread quantization involves rounding. The formulas for mid-tread uniform quantization are provided in the previous section.

Mid-riser quantization involves truncation. The input-output formula for a mid-riser uniform quantizer is given by:

Where and the construction rule is .

Note that mid-riser uniform quantizers do not have a zero output value – their minimum output magnitude is half the step size. In contrast, mid-tread quantizers do have a zero output level. For some applications, having a zero output signal representation may be a necessity.

Figure 1. Quantiser Input vs Output

In general, a mid-riser or mid-tread quantizer may not actually be a uniform quantizer – i.e., the size of the quantizer's classification intervals may not all be the same, or the spacing between its possible output values may not all be the same. The distinguishing characteristic of a mid-riser quantizer is that it has a classification threshold value that is exactly zero, and the distinguishing characteristic of a mid-tread quantizer is that is it has a reconstruction value that is exactly zero.

Consider , the effect of mid-rise and mid-tread quantisers on the outputs are shown below.

Figure 2. Effect of Different Types of Quantiser

In this case, the partition boundary values and codebook values are shown below.

Table I. partition boundary values and codebook values

|  | Partition Boundary | Code Book |
| --- | --- | --- |
| Mid-rise | [-1 -0.5 0 0.5 1] | [-1 -0.75 -0.25 0.25 0.75 1] |
| Mid-tread | [-1 -0.75 -0.25 0.25 0.75 1] | [-1 -1 -0.5 0 0.5 1 1] |

In Matlab, the function quantiz can be used for signal quantisation.